Identify quality-control measures/techniques

Explain lean operations.

There are nearly as many definitions of lean as there are lean practitioners and consultants. Nonetheless, a few basic principles are common to most, if not all, definitions. In short, the aim of any lean initiative is to eliminate waste.

- Transportation. Any required transportation of products or parts adds nothing from the customer's perspective. Therefore, it should be minimized or eliminated. It's common for plant redesign or reorganization to be among the first steps in a lean effort.

- Overproduction . This occurs when a part or product is manufactured before it's needed--either before a customer has ordered it or before it can begin the next process in the production system. This results in a backlog of material that must be stored, which adds unnecessary expense and inefficiency. A truly lean enterprise isn't dependent upon speculative market forecasting for determining production runs.

- Work in progress. WIP can be caused by overproduction, poor scheduling or long and uneven cycle times. Lean companies ensure that each manufacturing process takes roughly the same amount of time by adjusting the processes themselves or the resources dedicated to each. Additionally, suppliers must be held accountable for reliable delivery of their shipments.

Discuss the use of Six Sigma.

Six Sigma is a business initiative, not a quality initiative.  Six Sigma is an umbrella term for a philosophy and way of running a business that improves quality and productivity and increases profits.  There are three major components to Six Sigma:

- The environment or culture of the organization as established by the senior leaders in the organization

- The improvements tools such as SPC, design of experiments, process mapping, data display and analysis, problem-solving, and root cause analysis, and

- The infrastructure which includes systems that support the use of the tools.

The reason that companies should use Six Sigma is to increase profitability.  Sometimes however, the impetus to get started on a Six Sigma process is brought on by customer demands.

Six Sigma as a philosophy will create a heightened awareness of the need for on-going and everlasting quality improvement efforts.  Six Sigma as a process will provide employees with the tools and skills they need to embrace the Six Sigma philosophy.  By striving to achieve a six sigma level of quality, companies reduce costs related to scrap, rework, inspection, and customer dissatisfaction.

Describe the use of total quality management.

In the 1980s to the 1990s, a new phase of quality control and management began. This became known as Total Quality Management (TQM). Having observed Japan’s success of employing quality issues, western companies started to introduce their own quality initiatives. TQM, developed as a catchall phrase for the broad spectrum of quality-focused strategies, programs and techniques during this period, became the centre of focus for the western quality movement.

A typical definition of TQM includes phrases such as: customer focus, the involvement of all employees, continuous improvement and the integration of quality management into the total organisation. Although the definitions were all similar, there was confusion. It was not clear what sort of practices, policies, and activities needed to be implemented to fit the TQM definition.

Total Quality Management (TQM) is a management philosophy and operating approach that aims to consistently exceed the current and future expectations of all stakeholders (i.e., customers, employees, shareholders and the community). TQM is based on continuous improvement in all processes, goods and services, as a result of the creative involvement of all stakeholders.

Explain the use of continuous improvement.

Continuous improvement is constantly adapting by getting and using information, and by evaluating changes to make sure they were effective. It requires:

- Good information about your environment, from a variety of sources, to evaluate your outcomes (what you do) and your processes (how you do it).

- The ability to pull people together from different levels and areas to freely discuss the information and issues involved, come up with ideas, evaluate them, choose some, and carry them out.

- Systematic ways to measure your progress and the outcomes of your changes.

- A real desire to do it, even if it means changing power relationships and doing some things you do not completely agree with.

Discuss the use of statistical process improvement.

SPC is the primary analysis tool of quality improvement. It is the applied science that helps you collect, organize and interpret the wide variety of information available to your business. Whether you track revenues, billing errors, or the dimensions of manufactured components, SPC can help you measure, understand and control the variables that affect your business processes.

SPC analyzes the variation in whatever process you are measuring:

- First, control charts demonstrate how consistently your process is performing, and whether you should, or should not, attempt to adjust it.

- Next, the statistical process control chart compares the process performance to your customers' requirements, providing a process capability index as an ongoing, accurate direction for quality improvement.

- Finally, control charts and its resulting process capability index quickly evaluate the results of quality initiatives designed to improve process consistency.

Statistical Process Control is easy to do. Although it involves complex mathematics, computers are ideally suited to the task. They easily collect, organize and store information, calculate answers, and present results in easy to understand graphs, called control charts. Computers accept information typed in manually, read from scanners or manufacturing machines, or imported from other computer databases. The resulting control charts can be examined in greater detail, incorporated into reports, or sent across the Internet. A computer collecting information in real time can even detect very subtle changes in a process, and even warn you in time to prevent process errors before they occur.

SPC can help you understand and reduce the variation in any business process. Greater consistency in fulfilling your customer's requirements leads to greater customer satisfaction. Reduced variation in your internal processes leads to less time and money spent on rework and waste. Both directly yield greater profitability and security for your business. SPC is one of the essential tools necessary to maintain an advantage in today's competitive marketplace.